Name: __________________________________ Date: ___________________

Year: ________ Make: ____________ Model: ____________ Mileage: ________

VIN: _________________________________________________________

Automotive Service Technology Job Sheet

Pre-Alignment Inspection

Meets NATEF Task: (A4-C-1) Perform pre-alignment inspection and measure vehicle ride height; perform necessary action. (P-1).

1. Check service information for the specified tests and procedures to follow when performing a pre-alignment inspection. Describe the specified procedures:

______________________________________________________________________________

______________________________________________________________________________

2. Check tires. Both front tires and both rear tires should be checked for the following:

A. Correct tire pressure

B. Same size and brand

C. Same tread depth

OK _____ NOT OK _____

3. Perform a dry-park test to check for any looseness in the steering and suspension components such as:

A. Tie rods

B. Idler arms

C. Ball-joints

D. Control arm bushings

E. Loose or defective wheel bearings

OK _____ NOT OK _____

4. Check for proper ride height.

A. Front and rear                          B. Left and right                    OK _____ NOT OK _____

1 = Demonstrates exposure/observation of the competency

2 = Applies the competency but only mastered a few essential

attributes of the competency

3 = Capable of the competency but needs further practice

4 = Performs the competency satisfactorily

5 = Masters the competency					Instructor OK _______